1. Initialization and Priority Check

The program constantly scans the system from the end of the line back to the start. It prioritizes moving the batch furthest down the line (the Dryer) to the final position to prevent bottlenecks. If multiple stations are ready, it handles the one closest to the exit first.

2. Pickup Sequence

- The hoist moves horizontally to the target station (either the Start Sensor for a new load or a Bath/Washer that has finished its 10-second timer).

- Once aligned, the hoist lowers until it hits the lower limit sensor.

- The grab mechanism (magnet or claws) activates to secure the batch.

- The hoist lifts back up to the upper limit sensor to ensure it clears the tank walls.

3. Transfer and Deposit

- The hoist moves horizontally to the next station in the sequence (e.g., from Chemical Bath 1 to Washer 1).

- Upon arrival, the hoist lowers the batch into the tank or dryer.

- The grab mechanism deactivates, releasing the load.

- The hoist lifts back to the top position, leaving the batch behind.

4. Processing and Timing

- As soon as the hoist releases the load and moves up, the 10-second timer for that specific station begins.

- The visualization turns the station Red to show it is occupied.

- While the timer counts down, the hoist is "Idle" and immediately looks for other batches to move elsewhere in the line.

5. Completion

- Once the batch reaches the Dryer and finishes its 10 seconds, the hoist picks it up one last time.

- It moves to the End Position and lowers the load.

- The End Sensor triggers, the Completed Counter increments by one, and the hoist returns to the top to start the cycle over.